GT4 ST165 ECU PINOUT DIAGRAM

Revision 5 – 31/08/05

Connections shown in the same orientation as the ECU.

Notes

- Items highlighted in red are suspect and need to be verified. Do you have the answer to the missing info? Please e-mail me!

- This article is for the 1989 3S-GTE engine

- Downloaded from – www.gtfours.co.uk

- E-mail – kris@gtfours.co.uk

Bottom row of pins on the ECU

| Abbreviation | Wire colour | Means... | What does it do? | Anything else? |
| --- | --- | --- | --- | --- |
| E01 | White/Black | Earth | Attached to engine block |  |
| ISC-1 | Green/White | Idle Stabilisation Control | Opens a valve below the throttle body to control the idle speed rpm |  |
| STA | Black | Starting signal | 12V fed to the ECU whilst the car is trying to start |  |
| STJ | Green | Cold start injector | Signal to control the 5th injector when starting the car |  |
| #1 | Red/Black | Injector signal | Fires No 1 fuel injector |  |
| #2 | Light Green | Injector signal | Fires No 2 fuel injector |  |
| G- | Red | Distributor signals | Ground for G1, G2 and NE |  |
| G1 | Dark Green | Distributor signals | Sends crankshaft angle information | Allows ECU to determine the position of each piston, adjust injector and ignition timing |
| G2 | White | Distributor signals | Sends crankshaft angle information | As above, but 180o out from G1 |
| NE | Black | Engine rpm signal | Sends the engine speed and crankshaft angle to the ECU | Positive signal from 24 tooth sensor |
| FPR | Blue/Red | Fuel Pump Relay | Clicks on the fuel relay when you turn the ignition key | If it doesn't activate the fuel replay the car won't start |
| IGF | White/Red | Spark confirmation signal | Used to detect a fault within the igniter |  |
| THW | Red | Water temperature sensor | Informs ECU of water temperature | Probably used to correct fuel and ignition maps |
| WIN | Light Green/Black | Intercooler computer signal | Error reporting? |  |
| TPC | Black/White | TVSV signal | Allows extra boost pressure in 3+ gears ~2psi | Don't cut this wire, pull the pipe off from the wastegate actuator instead! |
| SPD | Blue/White | Speed sensor | Monitors car speed |  |
| STP | Green/White | Stop light switch | Monitors the brake light switch | To inform the ECU when braking, to apply deceleration fuel cut |
| THA | Grey | AFM air intake temp | Tells the ECU the temp of the incoming air |  |
| VS | Grey/Black | AFM signal | Stopped – idle signal |  |
| VC | Pink/Blue | AFM signal | Variable – flap position signal |  |
| BATT | Pink | Battery supply | Supplies power to ECU via the 15A engine bay fuse | Permanent 12V feed to the battery even when the ignition is off |
| ELS | Orange | Electrical idle up | Increases idle speed due to large current drawn by headlights and demister |  |

Top row of pins on the ECU

| Abbreviation | Wire colour | Means... | What does it do? | Anything else? |
| --- | --- | --- | --- | --- |
| E02 | White/Black | Earth | Attached to engine block |  |
| ISC-2 | Green/Black | Idle Stabilisation Control | Opens a valve below the throttle body to control the idle speed rpm |  |
| IGT | White | Igniter signal | Control spark to spark plugs |  |
| E1 | Brown | Earth | Attached to engine block |  |
| TVIS | Light Green | Operates TVIS VSV | Opens a set of butterflies in the intake manifold at 4300rpm |  |
| #3 | Yellow | Injector signal | Fires No 3 fuel injector |  |
| #4 | Blue | Injector signal | Fires No 4 fuel injector |  |
| VF | Violet/Yellow | Diagnostic connector | Check connector? | Any connections off? |
| T | Orange | Diagnostic connector | Check connector? | Any connections off? |
| VTA | White | Throttle position sensor | Informs ECU of throttle position | Very fine setup of the TPS sensor is required |
| IDL | Pink | Idle signal | Tells the ECU the car is at idle | Signal is from throttle position sensor |
| HT | Red/Green | To lambda sensor | 12V lambda sensor heater |  |
| OX | White | Lambda sensor voltage | A voltage feedback from the exhaust to "tune" the engine whilst at a steady rpm | Also feeds the OX pin at the diagnostic connector |
| E2 | Brown | Earth |  |  |
| E22 | Brown | Earth for AFM |  |  |
| EGR | Blue/Red | EGR VSV | Operates the EGR |  |
| A/C | Black/White | Clutch relay signal | Engages the AC compressor magnetic clutch |  |
| W | Green | Check engine light | Puts on the yellow engine check light | Indicates via a series of codes any stored faults |
| FPU | Black/Red | FPR control | Sends a signal to the fuel pressure VSV to increase the fuel pressure |  |
| KNK | Clear | Knock sensor | Sensor feeds in a voltage to the ECU proportional to the amount of knock in the engine... | If knock is sensed then the ignition timing is retarded |
| PIM | Red/White | TPS voltage | Feeds a voltage to the ECU proportional to the boost pressure | This signal is altered with an FCD if you want to run higher than stock boost. Also controls the dash boost gauge |
| THG | Blue | EGR temp sensor | Not fitted in UK | Only fitted to USA cars |
| +B | Black/Yellow | 12V supply to ECU | 12V fed after ignition key is turned |  |
| +B1 | Black/Yellow | 12V supply to ECU | 12V fed after ignition key is turned | Good place to attach items that you don't want to stay on after engine turned off e.g. BG light |